2009/06/30

Lefebure NTRIP Caster/Server Documentation

http://lefebure.com/software/

Background: NTRIP (Network Transport of RTCM via Internet Protocol) is a protocol designed for moving RTK correction data over the Internet. In the past, that data was commonly transmitted via radio links. Radios have distance limitations in some areas due to hills, trees, or other obstructions that block the radio signal. However, cell phone providers have been able to overcome these issues, and some RTK people looked at using the cell phone data network to transport this data. NTRIP was created, and is based off of a protocol that is used to transport streaming audio over the Internet.

Components: In an RTK system, there is a base station that monitors the GPS satellites. That base station then generates some correction data that can be used by rover receivers. Multiple rovers can use the same correction data, so one-to-many radio systems were commonly used. With NTRIP, we completely replace the radio transmitters and receivers with Internet connected equipment. There are 3 components in a NTRIP system – a Server, a Caster, and a Client. The Caster is in the center, and it brokers the data coming from one or more servers, and going to one or more clients. The Server gets data from a serial port and sends it to the Caster. The Client connects to the Caster and requests a certain data stream, also known as a mount point. Each base station generates a data stream, and most Casters allow multiple data streams to be available to the clients. Thus a client could request one of several available streams depending on location or stream format.

Purpose: This software is intended primarily to be a NTRIP Caster. It also includes a NTRIP Server in case you have a base station close to the computer. If your base station is farther away, you can use other NTRIP Server products to send the correction data in to the Caster. The GNSS Internet Radio Server is one product that can be used for this, but there are many others.

Limitations: The Lefebure NTRIP Caster/Server has no limitations on streams, users, or concurrent connections. Although the theoretical limitations are quite high, your first problem will likely be Internet bandwidth.

Computer Requirements: The computer hosting the NTRIP Caster needs to be Windows 2000 or newer, preferably 512MB of RAM or more, and have a reliable Internet connection. It will also need to have the .Net framework installed, which is a free download from Microsoft.

Internet Requirements: The Internet connection you use should be reliable and always on. It should also have a static IP address. The bandwidth requirement for each connection is related to what format the correction data is in. Some formats are more efficient than others. A good estimate is 1KB per second per connection.